Use at your own risk. The pilot must refer to the FAA approved documents for the

correct information and is responsible for safe operation of aircraft.

| Cessna 182P N9989E v1.8 8/28/2024 |  |
| --- | --- |
| TAKEOFF Normal: Rotate 60 MPH Vy 89 MPH (85 mph @ 10,000’) Short/Soft: Flaps 20° Vx 60 MPH (64 mph @ 10,000’) |  |
| CRUISE CLIMB 100 – 120 MPH BEST GLIDE 80 MPH (flaps up) |  |
| >1000’ AGL Oil Pres/Temp Ammeter Vacuum MP 15”- 23” Prop 2200 -2450 RPM |  |
| CRUISE 75% power or less LEAN to peak EGT, then ENRICH 75˚F rich of peak for best economy (125˚F for best power). Use G3X Engine Lean Assist OPERATING LEAN OF PEAK IS NOT APROVED Cowl Flaps - As Required Routinely Check Gauges: Oil Press/Temp Ammeter Landing Light On for Safety |  |
| LANDING Landing Light ON AutoPilot OFF GUMPS Carb Heat ON Prop FWD Cowl Flaps CLOSED Fuel BOTH Mixture ENRICH Approach at 70-80 MPH |  |
| GO AROUND Full Throttle Pitch up Carb Heat OFF Flaps retract to 20° Climb at 60 mph & above 80 mph retract flaps Cowl Flaps OPEN |  |
| AFTER LANDING Carb Heat OFF Transponder SBY Flaps UP Cowl Flaps OPEN Mixture LEAN for taxi |  |
| SHUTDOWN Avionics/Lights OFF G3X Engine & Total Times RECORDED Mixture Mags Master OFF Leave Beacon On Flight Plan Closed Control Lock Installed Pitot Tube Covers - Installed |  |
| POSTFLIGHT Windows & All Leading Edges CLEANED Wheel Chocks SET Seat Belts SECURE Interior CLEANED Airplane LOCKED LOG OUT Flight Log System Flight Plan CLOSED Hangar SECURE Lights OFF Doors LOCKED |  |
| MPH Vso 63 Vs1 68 Vfe 110 Va 2950 126 Va 2500 116 Va 2050 105 Vno 160 Vne 198 | Recommended Pattern Speeds/ Approx Pwr Settings Downwind: 90 - 100 mph / 16“ MP Abeam: 85 - 90 mph / 13“ MP Base: 80 - 85 mph / 13“ MP Final: 70 - 75 mph / 13“ MP (Full Flaps – No Flaps) |

Emergency:  Com   121.5 / Squawk 7700

Loss Com.  Squawk 7600

KMAN ASOS:118.325    KEUL COM:123.0 / ASOS:135.07

KBOI ATIS:123.9  Big Sky App/Dep: 119.6 unless directed to 126.9

Use at your own risk. The pilot must refer to the FAA approved documents for the

correct information and is responsible for safe operation of aircraft.

| Cessna 182P N9989E v1.8 8/28/2024 |  |
| --- | --- |
| ## CHECKED OUT - FLIGHT LOG SYSTEM ## PREFLIGHT Pitot/AOA Tube Covers-REMOVED Gust Lock - REMOVED Avionics Master–OFF AOA Pitot Heat/Pitot Heat - OFF Master – ON (Right side only) G3X – Check Engine(tach) Total (Hobbs) Flaps – Ck for Smooth Operation Exterior/Interior Lights - CHECK Stall Horn - CHECK Master – OFF Baggage Door – LOCKED Tires/Brakes – Press/Leaks (Main 42 psi - Nose 49 psi) Oil level: 7 MIN / 8 MAX Qts. Oil Filler Cap – Secure Nose strut, Cowl Flaps-Inspect Prop & Spinner-Inspect Static Ports-Clear Check Fuel level/Caps/Drain Engine Fuel Strainer-Drain Final Walk/Look Around/Tow Bar ## START Passenger Briefing Seats – LOCKED Seat Belts – FASTENED Fuel – ON BOTH Cowl Flaps – OPEN Elevator / Rudder Trim-Set Circuit Breakers – In Master – ON Wait until G3X is on and MFD/Oil Pressure is visible. Set Remaining Fuel Qty to match dipstick. Reset Set Fuel Used = 0 AutoPilot Preflight (if intending to use AutoPilot or Electric Trim) See G3x manual pg 295 and FMS-Limitations | Mixture – RICH Carb Heat – COLD Prime – As Required (max 3) Throttle – OPEN ~ ½ Inch Brakes – Set/Held Beacon – ON Yell - Clear Prop – Look around ## Engage Starter (max 20 sec crank if no start) ## AFTER START Throttle – 1000 RPM Oil Pressure– in green < 30 seconds Flaps – UP Mixture – LEAN FOR TAXI Avionics Master – ON Taxi/Landing Lights - ON Transponder – ALT ## RUNUP Doors & Windows – LATCHED Controls – FREE & CORRECT Flaps – UP Trim – Elevator & Rudder TAKEOFF Throttle – 1700 RPM Mixture – FULL RICH (Lean for best power above 3,000’ DA) Mags (drop< 150, difference ea <50) Prop – Cycle 3 X Carb Heat – TEST Throttle to Idle – Does not quit Throttle – 1000 RPM Confirm Voltage 12.5-14 Volts Hi Voltage Light – NOT LIT Check Alt Amps - Charging Instruments – CHECK/SET Seat Belts – SECURE Headset – ON Check Intercom/Com volumes/operation COM – First Freq. 122.7 KMAN KMAN ASOS:118.325 AutoPilot – OFF G3X/Avionics – Set Up |
| BEFORE TAKEOFF - Clearing circle for Pattern Traffic What is your ABORT Plan? Flaps UP or 20° Carb Heat OFF Transponder ALT Lights/Strobes ON Prop FWD Cowl Flaps OPEN Headset ON |  |

Emergency:  Com   121.5 / Squawk 7700

Loss Com.  Squawk 7600

KMAN ASOS:118.325    KEUL ASOS :135.07     KBOI ATIS:123.9

Big Sky App/Dep:  North 126.9 / South 119.6